Product Requirements Document

C-ITS PKI Implementation

Certificate Generation conforming to ETSI TS 103 097 V2.2.1

| Document version: | 1.0 |
| --- | --- |
| Status: | Draft |
| Date: | March 20, 2026 |
| Standard reference: | ETSI TS 103 097 V2.2.1 (2026-03) |
| Base standard: | IEEE Std 1609.2™-2025 |

# 1. Executive Summary

This Product Requirements Document (PRD) specifies the functional and technical requirements for implementing a simple Public Key Infrastructure (PKI) capable of generating and managing digital certificates for Cooperative Intelligent Transport Systems (C-ITS). The implementation shall conform to ETSI TS 103 097 V2.2.1 — "Intelligent Transport Systems (ITS); Security; Security header and certificate formats; Release 2" — which itself is defined as a profile of IEEE Std 1609.2™-2025.

The PKI system must support the full certificate hierarchy required by the C-ITS trust model: a self-signed Root Certification Authority (Root CA), subordinate Enrolment Authority (EA) and Authorization Authority (AA) certificates, as well as end-entity Enrolment Credentials (EC) and Authorization Tickets (AT) issued to ITS-Stations. A Trust List Manager (TLM) certificate is also required. All certificates shall use the EtsiTs103097Certificate format encoded with COER (Canonical Octet Encoding Rules) as mandated by the specification.

# 2. Background and Context

Cooperative Intelligent Transport Systems rely on vehicles, roadside units, and infrastructure nodes exchanging safety-critical messages such as Cooperative Awareness Messages (CAMs) and Decentralized Environmental Notification Messages (DENMs). These messages must be digitally signed to ensure authenticity and integrity, and optionally encrypted for confidentiality. The trust framework enabling this relies on a multi-level PKI that issues short-lived pseudonym certificates (Authorization Tickets) to ITS-Stations so that vehicle identity is protected while message authenticity is guaranteed.

ETSI TS 103 097 V2.2.1 defines the precise data structures, certificate formats, cryptographic algorithms, and security profiles that all conformant C-ITS implementations must use. It builds upon IEEE Std 1609.2 as the base data format standard and adds ETSI-specific profiling constraints.

## 2.1 Normative References

| Ref. | Document |
| --- | --- |
| [1] | IEEE Std 1609.2™-2025 — Wireless Access in Vehicular Environments – Security Services for Application and Management Messages |
| [2] | ETSI TS 102 965 — ITS Application Object Identifier (ITS-AID); Registration; Release 2 |
| [3] | ITU-T Recommendation X.696 — ASN.1 Canonical Octet Encoding Rules (COER/OER) |

## 2.2 Informative References

| Ref. | Document |
| --- | --- |
| [i.1] | ETSI TS 102 940 — ITS Communications Security Architecture and Security Management; Release 2 |
| [i.2] | ETSI TS 102 941 — ITS Security; Trust and Privacy Management; Release 2 |
| [i.3] | ETSI TS 103 601 — ITS Security; Security Management Messages Communication Requirements and Distribution Protocols; Release 2 |
| [i.4] | CPOC Protocol |
| [i.5] | ETSI TS 103 096-1 — ITS Testing; Conformance Test Specifications for ITS Security Part 1 (PICS); Release 2 |

# 3. Scope

## 3.1 In Scope

The following capabilities are within the scope of this implementation:

Root CA certificate generation (self-signed, explicit certificate type)

Enrolment Authority (EA) subordinate CA certificate generation

Authorization Authority (AA) subordinate CA certificate generation

Trust List Manager (TLM) certificate generation

Enrolment Credential (EC) issuance for ITS-Stations

Authorization Ticket (AT) issuance for ITS-Stations

EtsiTs103097Certificate format conformance (clause 6 and 7.2 of ETSI TS 103 097)

ECDSA signing with NIST P-256 and P-384 curves

AES-128-CCM symmetric encryption and ECIES key wrapping

COER (Canonical OER) encoding of all ASN.1 data structures

Signed data structure generation conforming to EtsiTs103097Data-Signed profile

Encrypted data structure generation conforming to EtsiTs103097Data-Encrypted profile

Certificate validity period management

Permissions management (appPermissions and certIssuePermissions)

Hash ID-based revocation support for EA and AA certificates

## 3.2 Out of Scope

The following are explicitly out of scope for this implementation:

Online Certificate Status Protocol (OCSP)

Full CPOC (Central Point of Contact) protocol implementation

Certificate Revocation List (CRL) distribution infrastructure (format defined in ETSI TS 102 941)

Certificate Trust List (CTL) distribution infrastructure

Misbehaviour Detection Management (MDM) processing logic

V2X radio communication stack

ITS-S on-board application integration

Web-based management portal or GUI

# 4. Glossary and Abbreviations

| Abbreviation / Term | Definition |
| --- | --- |
| AA | Authorization Authority — subordinate CA that issues Authorization Tickets |
| ASN.1 | Abstract Syntax Notation One — formal notation for data structure schemas |
| AT | Authorization Ticket — short-lived pseudonym certificate issued to an ITS-Station |
| CAM | Cooperative Awareness Message — periodic V2X safety broadcast |
| C-ITS | Cooperative Intelligent Transport Systems |
| COER | Canonical Octet Encoding Rules — the required ASN.1 encoding per ITU-T X.696 |
| CPOC | Central Point of Contact — operator coordinating the ETSI C-ITS PKI |
| CTL | Certificate Trust List — signed list of trusted Root CA certificates |
| CRL | Certificate Revocation List |
| DENM | Decentralized Environmental Notification Message — event-driven V2X message |
| EA | Enrolment Authority — subordinate CA that issues Enrolment Credentials |
| EC | Enrolment Credential — long-term identity certificate for an ITS-Station |
| ECDSA | Elliptic Curve Digital Signature Algorithm |
| ECIES | Elliptic Curve Integrated Encryption Scheme |
| ITS | Intelligent Transport Systems |
| ITS-AID | ITS Application Identifier — registered per ETSI TS 102 965 |
| ITS-S | ITS-Station (vehicle OBU, RSU, or back-end node) |
| KDF | Key Derivation Function |
| MDM | Misbehaviour Detection Management |
| OBU | On-Board Unit |
| OID | Object Identifier |
| PICS | Protocol Implementation Conformance Statement |
| PKI | Public Key Infrastructure |
| Root CA | Root Certification Authority — the trust anchor of the hierarchy |
| RSU | Road-Side Unit |
| TLM | Trust List Manager — entity managing the Certificate Trust List |
| V2X | Vehicle-to-Everything communications |

# 5. System Architecture Overview

The C-ITS PKI follows a three-tier hierarchy. The Root CA is the trust anchor and signs subordinate CA certificates. Two categories of subordinate CA exist: the Enrolment Authority (EA), which issues long-lived Enrolment Credentials to ITS-Stations, and the Authorization Authority (AA), which issues short-lived, pseudonymous Authorization Tickets. A Trust List Manager (TLM) certificate establishes the signed Certificate Trust List. Each certificate type must conform to a specific security profile defined in clause 7.2 of ETSI TS 103 097 V2.2.1.

## 5.1 PKI Hierarchy

| Entity | Certificate Type | Issuer | Issues |
| --- | --- | --- | --- |
| Trust List Manager (TLM) | TLM Certificate | Self (issuer = self) | Signs CTLs |
| Root CA | Root CA Certificate | Self (issuer = self) | EA Cert, AA Cert |
| Enrolment Authority (EA) | Sub-CA Certificate | Root CA | Enrolment Credentials |
| Authorization Authority (AA) | Sub-CA Certificate | Root CA | Authorization Tickets |
| ITS-Station (enrolment) | Enrolment Credential (EC) | EA | N/A — end entity |
| ITS-Station (pseudonym) | Authorization Ticket (AT) | AA | N/A — end entity |

## 5.2 Data Format Foundation

All data structures shall be defined using ASN.1 as specified in the normative ASN.1 module EtsiTs103097Module. The module imports from the IEEE Std 1609.2 modules Ieee1609Dot2 and Ieee1609Dot2BaseTypes. All encoding shall use COER as required by ITU-T X.696. The module OID is:

{itu-t(0) identified-organization(4) etsi(0) itsDomain(5) wg5(5)

secHeaders(103097) core(1) major-version-3(3) minor-version-2(2)}

# 6. Cryptographic Algorithm Requirements

The following cryptographic algorithms are mandated by ETSI TS 103 097 V2.2.1 clause 4.2. All implementations shall support these algorithms; no others shall be used for interoperability-critical operations.

| Algorithm Category | Algorithm | Standard Reference | Notes |
| --- | --- | --- | --- |
| Digital Signature | ECDSA | IEEE Std 1609.2 §5.3.1 | P-256 (SHA-256) or P-384 (SHA-384) |
| Hash (signature) | SHA-256 | IEEE Std 1609.2 | Used with P-256 curve |
| Hash (signature) | SHA-384 | IEEE Std 1609.2 | Used with P-384 curve |
| Public Key Encryption | ECIES | IEEE Std 1609.2 §5.3.5 | Elliptic Curve Integrated Encryption Scheme |
| Symmetric Encryption | AES-128-CCM | IEEE Std 1609.2 §5.3.8 | 16-octet key; 12-octet nonce; CCM mode |
| Key Derivation | KDF2 based on SHA-256 | IEEE Std 1609.2 | TRUNCATE(SHA256(S¦¦0x00000001¦¦P1) ¦¦ SHA256(S¦¦0x00000002¦¦P1), 48) |
| MAC | HMAC based on KDF2 output | FIPS PUB 198-1 | km = last 32 bytes of KDF2; MAC(m,km) = HMAC(m,km) |

## 6.1 ECIES Key Encapsulation Details

The ECIES scheme used for key wrapping shall implement the following parameters:

Elliptic curve: NIST P-256 (or P-384 for SHA-384 profiles); base point G, order q

Sender generates ephemeral private key r ∈ [1, q-1]; ephemeral public key V = r·G (33 octets, compressed)

Shared secret: S = Px, where (Px, Py) = r·Kr and Kr is the receiver's public encryption key

KDF output: (ke ‖ km) = KDF2(S), where ke is 16 octets and km is 32 octets

AES key encryption: c = E(A, ke) using XOR — c = A ⊕ ke (16 octets)

Authentication tag: t = MAC(c, km) = HMAC(c, km), 16 octets

## 6.2 AES-CCM Message Encryption

Key A: 16 octets, randomly generated per message

Nonce n: 12 octets, randomly generated

AES-CCM encrypts message m producing ciphertext C = encrypted-m ‖ CCM-authentication-tag

Transmitted alongside: V (ephemeral public key), c (encrypted AES key), t (ECIES auth tag), nonce n, cert_id (8 octets)

# 7. Secure Data Structure Requirements

A secure data structure shall be of type EtsiTs103097Data, corresponding to Ieee1609Dot2Data as defined in IEEE Std 1609.2, clause 6.3.2, with the constraints defined in clauses 5.1–5.3 of ETSI TS 103 097 V2.2.1.

## 7.1 Supported Data Structure Variants

| Type Name | Description | Clause |
| --- | --- | --- |
| EtsiTs103097Data-Unsecured | Wraps unsecured (plain) data payload | 5.1 |
| EtsiTs103097Data-Signed | Signed message; payload in tbsData.payload.data | 5.1, 5.2 |
| EtsiTs103097Data-SignedExternalPayload | Signed message with external payload; digest in tbsData.payload.extDataHash | 5.1, 5.2 |
| EtsiTs103097Data-Encrypted | AES-CCM encrypted payload in ciphertext.aes128ccm | 5.1, 5.3 |
| EtsiTs103097Data-SignedAndEncrypted | Signed message wrapped inside an Encrypted structure | 5.1 |
| EtsiTs103097Data-Encrypted-Unicast | Encrypted with exactly one entry in recipients | 5.1 |
| EtsiTs103097Data-SignedAndEncrypted-Unicast | Signed and encrypted with exactly one recipient | 5.1 |

## 7.2 SignedData Constraints

The type SignedData shall conform to the following constraints (clause 5.2):

hashId: indicates the hash algorithm (sha256 or sha384) per IEEE Std 1609.2 clauses 6.3.5 and 5.3.3

tbsData shall be of type ToBeSignedData (IEEE Std 1609.2 clause 6.3.6)

tbsData.payload shall be of type SignedDataPayload containing either data (Ieee1609Dot2Data) or extDataHash

tbsData.headerInfo.psid: shall contain the ITS-AID for the relevant message type (registered in ETSI TS 102 965)

tbsData.headerInfo.generationTime: always present

tbsData.headerInfo.expiryTime: present or absent per message security profile (clause 7.1)

tbsData.headerInfo.generationLocation: present or absent per message security profile

tbsData.headerInfo.p2pcdLearningRequest: always absent

tbsData.headerInfo.missingCrlIdentifier: always absent

tbsData.headerInfo.encryptionKey: present or absent per message security profile

signer shall be of type SignerIdentifier, constrained to: (a) digest — 8-octet hash of the signing AT/EC certificate, or (b) certificate — a single EtsiTs103097Certificate

signature shall contain the ECDSA signature per IEEE Std 1609.2 clauses 6.3.38, 6.3.39 and 5.3.1

## 7.3 EncryptedData Constraints

The type EncryptedData shall conform to the following constraints (clause 5.3):

recipients shall be of type SequenceOfRecipientInfo; each entry shall be pskRecipInfo, certRecipInfo, or signedDataRecipInfo

Encryption scheme: ECIES as defined in IEEE Std 1609.2 clause 5.3.5

ciphertext shall be of type SymmetricCiphertext containing an EtsiTs103097Data encrypted per AES-CCM

The inner encrypted payload shall be an EtsiTs103097Data structure

# 8. Certificate Format Requirements

All certificates shall be of type EtsiTs103097Certificate corresponding to a single Certificate as defined in IEEE Std 1609.2 clause 6.4.2, with the constraints defined in clause 6 of ETSI TS 103 097 V2.2.1. A certificate shall be either explicit (verifyKeyIndicator = verificationKey) or implicit (verifyKeyIndicator = reconstructionValue) at the discretion of the issuing entity. Receivers shall be able to validate both types.

## 8.1 toBeSigned Component Constraints

The component toBeSigned (of type ToBeSignedCertificate) shall be constrained as follows:

id: of type CertificateId, constrained to choice name (for CA and EC certificates) or none (for AT certificates)

cracaId: set to 000000H (revocation via Hash ID method defined in ETSI TS 102 941)

crlSeries: set to 0D

validityPeriod: the validity interval [start, start + duration) — start inclusive, end exclusive; no further constraints

region: of type GeographicRegion, present or absent per certificate profile; the value 65535 designates the European Union (27 member states as of 31 January 2020)

assuranceLevel: of type SubjectAssurance, present or absent per certificate profile

appPermissions: of type SequenceOfPsidSsp, present or absent per certificate profile; at least one of appPermissions or certIssuePermissions shall be present

certIssuePermissions: of type SequenceOfPsidGroupPermissions, present or absent per certificate profile

certRequestPermissions: always absent

canRequestRollover: always absent

encryptionKey: of type PublicEncryptionKey, present or absent per certificate profile

verifyKeyIndicator: verificationKey if explicit certificate; reconstructionValue if implicit

flags: present or absent per [i.2]

appExtensions: present only if AppExtension OperatingOrganizationId identified by certExtId-OperatingOrganization is present

certIssueExtensions: absent, or optionally present only if appExtensions is present

certRequestExtensions: absent, or optionally present only if certIssueExtensions is present

## 8.2 Signature Component

The component signature of EtsiTs103097Certificate shall be of type Signature as defined in IEEE Std 1609.2 clause 6.3.37, and shall contain the ECDSA signature calculated by the signer identified in the issuer component (clauses 6.3.38, 6.3.39, and 5.3.1 of IEEE Std 1609.2).

## 8.3 Certificate Validity Assessment

The validity of a certificate shall be assessed as defined in IEEE Std 1609.2 clause 5.1, using the Hash ID-based revocation method for EA and AA certificates. No revocation method applies to Authorization Tickets and Enrolment Credentials. Root CA certificate validity against the CTL is governed by ETSI TS 102 941.

# 9. Certificate Profile Requirements

Each certificate type shall implement a specific security profile as defined in clause 7.2 of ETSI TS 103 097 V2.2.1. The following sub-sections specify the mandatory constraints for each profile.

## 9.1 Root CA Certificate

Certificate type: explicit (verifyKeyIndicator = verificationKey)

issuer: set to self

CertificateId (toBeSigned.id): choice name — shall contain a unique name string for the root CA

certIssuePermissions: present — indicates issuing permissions (permissions to sign subordinate CA certificates)

appPermissions: present — shall include ITS-AID for the CRL service and ITS-AID for the CTL service (registered per ETSI TS 102 965)

encryptionKey: not required (absent)

Additional requirements: per CPOC protocol [i.4]

## 9.2 Enrolment Authority (EA) Certificate

Certificate type: explicit

issuer: sha256AndDigest or sha384AndDigest — digest of the signing Root CA certificate

CertificateId: choice name — unique name for the EA

certIssuePermissions: present — indicates permissions to sign Enrolment Credentials

appPermissions: present — indicates permissions to sign certificate response messages (EtsiTs103097Data)

encryptionKey: present — public encryption key for ITS-Stations to encrypt EC request messages to the EA

Additional requirements: per CPOC protocol [i.4]

## 9.3 Authorization Authority (AA) Certificate

Certificate type: explicit

issuer: sha256AndDigest or sha384AndDigest — digest of the signing Root CA certificate

CertificateId: choice name — unique name for the AA, or choice none

certIssuePermissions: present — indicates permissions to sign Authorization Tickets

appPermissions: present — indicates permissions to sign certificate response messages (EtsiTs103097Data)

encryptionKey: present — public encryption key for ITS-Stations to encrypt AT request messages to the AA

Additional requirements: per CPOC protocol [i.4]

## 9.4 Trust List Manager (TLM) Certificate

Certificate type: explicit

issuer: set to self

CertificateId: choice name — unique name string for the TLM

appPermissions: present — shall contain the ITS-AID for the CTL service (per ETSI TS 102 965)

encryptionKey: absent

certIssuePermissions: absent

Additional requirements: per CPOC protocol [i.4]

## 9.5 Enrolment Credential (EC)

Certificate type: explicit

issuer: sha256AndDigest or sha384AndDigest — digest of the signing EA certificate

CertificateId: choice name — unique name associated with the enrolment credential

appPermissions: present — indicates permissions to sign certificate request messages (EtsiTs103097Data)

certIssuePermissions: absent

The EC is the long-term identity credential for the ITS-Station; it is used to obtain ATs

## 9.6 Authorization Ticket (AT)

Certificate type: explicit or implicit — at the discretion of the AA

issuer: sha256AndDigest or sha384AndDigest — digest of the signing AA certificate

CertificateId: choice none (pseudonymous — no name)

appPermissions: present — indicates message signing permissions (permissions to sign an EtsiTs103097Data)

certIssuePermissions: absent

ATs are short-lived pseudonym certificates used for V2X message signing; no linkage to vehicle identity shall be inferable

## 9.7 Misbehaviour Authority Certificate

Certificate type: explicit

issuer: sha256AndDigest or sha384AndDigest

CertificateId: choice name — unique name for the misbehaviour authority

encryptionKey: present — public key for ITS-Stations to encrypt misbehaviour reports

certIssuePermissions: absent

appPermissions: present — shall include at least the ITS-AID for the Misbehaviour Detection Management (MDM) service

# 10. Message Security Profile Requirements

Clause 7.1 of ETSI TS 103 097 V2.2.1 defines security profiles for messages processed by the PKI. The PKI implementation shall generate and validate signed data structures conforming to the following profiles.

## 10.1 CAM Security Profile

Secure data structures containing Cooperative Awareness Messages shall conform to EtsiTs103097Data-Signed with:

signer: by default choice digest (8-octet hash of the AT); choice certificate (full AT) included once per second after last inclusion, and immediately upon receiving a CAM signed by an unknown AT

headerInfo.psid: ITS-AID value for CAMs as assigned in ETSI TS 102 965

headerInfo.generationTime: always present

headerInfo.inlineP2pcdRequest: shall be included when the ITS-S receives a digest pointing to an unknown AT, or receives a message with a certificate signed by an unknown AA certificate

headerInfo.requestedCertificate: included if the ITS-S finds a digest of a valid AA certificate in an inlineP2pcdRequest received from a peer

All other headerInfo components allowed by clause 5: absent

## 10.2 DENM Security Profile

Secure data structures containing Decentralized Environmental Notification Messages shall conform to EtsiTs103097Data-Signed with:

signer: choice certificate — shall contain a full EtsiTs103097Certificate

headerInfo.psid: ITS-AID value for DENMs as assigned in ETSI TS 102 965

headerInfo.generationTime: always present

headerInfo.generationLocation: shall be present

All other headerInfo components: absent

## 10.3 Generic Signed Message Profile

Signed messages other than CAM and DENM shall use either EtsiTs103097Data-Signed (full payload) or EtsiTs103097Data-SignedExternalPayload (digest only), with constraints from clause 5.2.

## 10.4 Encrypted Message Profile

Encrypted messages shall use type EtsiTs103097Data-Encrypted with constraints from clause 5.3.

## 10.5 Signed and Encrypted Message Profile

Messages that are both signed and encrypted shall use EtsiTs103097Data-SignedAndEncrypted. This is an EtsiTs103097Data-Encrypted containing an EtsiTs103097Data-Signed as the plaintext payload.

# 11. Functional Requirements

## 11.1 Key Pair Generation

| ID | Requirement |
| --- | --- |
| FR-KG-01 | The system shall generate ECDSA P-256 key pairs for use with SHA-256 |
| FR-KG-02 | The system shall generate ECDSA P-384 key pairs for use with SHA-384 |
| FR-KG-03 | Private keys shall be generated using a cryptographically secure random number generator |
| FR-KG-04 | Public keys shall be stored in compressed point format (33 octets for P-256, 49 for P-384) |
| FR-KG-05 | Separate key pairs shall be generated for signing and encryption for each PKI entity |

## 11.2 Certificate Issuance

| ID | Requirement |
| --- | --- |
| FR-CI-01 | The system shall generate self-signed Root CA certificates conforming to profile 9.1 |
| FR-CI-02 | The system shall generate EA certificates signed by the Root CA conforming to profile 9.2 |
| FR-CI-03 | The system shall generate AA certificates signed by the Root CA conforming to profile 9.3 |
| FR-CI-04 | The system shall generate TLM certificates (self-signed) conforming to profile 9.4 |
| FR-CI-05 | The system shall issue Enrolment Credentials signed by the EA conforming to profile 9.5 |
| FR-CI-06 | The system shall issue Authorization Tickets signed by the AA conforming to profile 9.6 |
| FR-CI-07 | All certificates shall be of type EtsiTs103097Certificate encoded in COER |
| FR-CI-08 | Certificate validity periods shall be configurable; the interval is [start, start+duration) |
| FR-CI-09 | At least one of appPermissions or certIssuePermissions shall be present in each certificate |
| FR-CI-10 | cracaId shall be set to 000000H and crlSeries to 0D in all certificates |
| FR-CI-11 | The system shall support both explicit and implicit certificate types and receivers shall validate both |

## 11.3 Signing Operations

| ID | Requirement |
| --- | --- |
| FR-SN-01 | The system shall produce SignedData structures of type EtsiTs103097Data-Signed |
| FR-SN-02 | Signatures shall use ECDSA per IEEE Std 1609.2 clause 5.3.1 |
| FR-SN-03 | hashId shall match the curve: sha256 with P-256, sha384 with P-384 |
| FR-SN-04 | The signer field shall be either digest (8-octet cert hash) or certificate (full EtsiTs103097Certificate) |
| FR-SN-05 | headerInfo.generationTime shall always be present in signed data |
| FR-SN-06 | p2pcdLearningRequest and missingCrlIdentifier shall always be absent |
| FR-SN-07 | The system shall support external payload signing (EtsiTs103097Data-SignedExternalPayload) |

## 11.4 Encryption Operations

| ID | Requirement |
| --- | --- |
| FR-EN-01 | The system shall produce EncryptedData structures using ECIES for key encapsulation |
| FR-EN-02 | Message payload shall be encrypted with AES-128-CCM (16-octet key, 12-octet nonce) |
| FR-EN-03 | The system shall support pskRecipInfo, certRecipInfo, and signedDataRecipInfo recipient types |
| FR-EN-04 | The system shall support unicast encryption (exactly one recipient) |
| FR-EN-05 | The ephemeral ECDSA key V shall be freshly generated per encryption operation |
| FR-EN-06 | The system shall implement the ECIES decryption procedure as specified in Annex B.3 |

## 11.5 Certificate Verification

| ID | Requirement |
| --- | --- |
| FR-VF-01 | The system shall verify certificate signatures using ECDSA |
| FR-VF-02 | Certificate validity shall be assessed per IEEE Std 1609.2 clause 5.1 |
| FR-VF-03 | The system shall verify SignedData signatures and reconstruct the message hash |
| FR-VF-04 | Revocation checking for EA and AA certificates shall use the Hash ID-based method |
| FR-VF-05 | The system shall correctly parse both explicit and implicit certificates |
| FR-VF-06 | The system shall implement region constraint checking; value 65535 = EU-27 |

## 11.6 Permissions Management

| ID | Requirement |
| --- | --- |
| FR-PM-01 | appPermissions shall encode ITS-AID values using PSID as defined in ETSI TS 102 965 |
| FR-PM-02 | certIssuePermissions shall encode PsidGroupPermissions constraining what sub-certificate permissions may be granted |
| FR-PM-03 | An AT shall carry appPermissions only (no certIssuePermissions) |
| FR-PM-04 | An EC shall carry appPermissions for certificate request message signing |
| FR-PM-05 | Root CA shall carry certIssuePermissions for signing subordinate CAs and appPermissions for CRL/CTL service AIDs |
| FR-PM-06 | EA and AA shall carry certIssuePermissions for signing end-entity certificates and appPermissions for certificate response message signing |

# 12. Non-Functional Requirements

## 12.1 Security

| ID | Requirement |
| --- | --- |
| NFR-SEC-01 | Private keys shall never be stored in plaintext outside of a secure element or HSM |
| NFR-SEC-02 | All random number generation shall use a CSPRNG meeting at least NIST SP 800-90A Rev. 1 requirements |
| NFR-SEC-03 | The system shall prevent timing side-channel attacks on ECDSA and ECIES operations |
| NFR-SEC-04 | AES-CCM nonces shall be unique per encryption operation; nonce reuse with the same key is prohibited |
| NFR-SEC-05 | Authorization Ticket private keys shall be independent of Enrolment Credential private keys |
| NFR-SEC-06 | The system shall not expose vehicle identity through AT certificate fields; AT id shall be choice none |

## 12.2 Interoperability

| ID | Requirement |
| --- | --- |
| NFR-INT-01 | All ASN.1 data structures shall be encoded using COER (ITU-T X.696) |
| NFR-INT-02 | The implementation shall conform to ETSI TS 103 097 V2.2.1 PICS proforma (ETSI TS 103 096-1) |
| NFR-INT-03 | The implementation shall correctly handle UnCountryId, CountryAndRegions, CountryAndSubregions, and RegionAndSubregions by parsing (not necessarily recognizing all values) |
| NFR-INT-04 | OID of the ETSI ASN.1 module shall match the value specified in Annex A.1 of the standard |
| NFR-INT-05 | The system shall be backwards compatible with explicit certificates from prior versions (V1.3.1 and V1.4.1) |

## 12.3 Performance

| ID | Requirement |
| --- | --- |
| NFR-PER-01 | AT issuance latency shall not exceed 500 ms per certificate under nominal conditions |
| NFR-PER-02 | CAM signing latency shall not exceed 10 ms for ECDSA P-256 signing |
| NFR-PER-03 | The system shall support concurrent certificate issuance requests (minimum 10 concurrent) |
| NFR-PER-04 | Certificate verification latency shall not exceed 50 ms per certificate chain |

## 12.4 Reliability and Availability

| ID | Requirement |
| --- | --- |
| NFR-REL-01 | The PKI backend (Root CA, EA, AA) shall target 99.9% availability during operational hours |
| NFR-REL-02 | Root CA key material shall be stored with hardware redundancy and offline backup |
| NFR-REL-03 | Certificate issuance operations shall be atomic — partial issuance failures shall not produce malformed certificates |

# 13. ASN.1 Encoding Requirements

All data structures shall be defined and encoded in accordance with the normative ASN.1 modules referenced in Annex A of ETSI TS 103 097 V2.2.1.

## 13.1 ASN.1 Module References

| Module Name | Module OID | SHA-256 Hash |
| --- | --- | --- |
| EtsiTs103097Module | {itu-t(0) id-org(4) etsi(0) itsDomain(5) wg5(5) secHeaders(103097) core(1) major-version-3(3) minor-version-2(2)} | 245a3c10c176497f658c6a4d9ec804d3226dda2ac10a4351e382ddb5afe9ff72 |
| EtsiTs103097ExtensionModule | {itu-t(0) id-org(4) etsi(0) itsDomain(5) wg5(5) secHeaders(103097) extension(2) major-version-1(1) minor-version-2(2)} | b94c9b373567dd9bfb15d61c8c206d5630f6b0c481ef41ddd574c96784a9eb1a |
| Ieee1609Dot2 | {iso(1) id-org(3) ieee(111) standards-assoc-numbered-series-stds(2) wave-stds(1609) dot2(2) base(1) schema(1) major-version-2(2) minor-version-7(7)} | 82b5e35cbaadae1c6b2f087626b10d52afc73779c9ac6700c6445830d8824e0b |
| Ieee1609Dot2BaseTypes | {iso(1) id-org(3) ieee(111) standards-assoc-numbered-series-stds(2) wave-stds(1609) dot2(2) base(1) base-types(2) major-version-2(2) minor-version-5(5)} | bf2b3d66d394449319323f8a59d8084d963c3ad55d6790e1436ab897221690fe |

## 13.2 Encoding Rules

All ASN.1 data structures shall be encoded using Canonical OER (COER) as defined in ITU-T X.696

For structures with a "canonical encoding" in IEEE Std 1609.2, that canonical encoding shall be used

The ASN.1 modules shall be obtained from the ETSI forge repository and integrity verified against SHA-256 hashes in Table 13.1

Extension mechanisms (ASN.1 extensibility) shall be supported per clause 4.3 of ETSI TS 103 097 V2.2.1

HeaderInfo extensions shall use the contributedExtensions component with etsiHeaderInfoContributorId = 2

# 14. Acceptance Criteria

The implementation shall be considered accepted when all of the following criteria are demonstrated through testing:

| AC ID | Acceptance Criterion | Test Method |
| --- | --- | --- |
| AC-01 | Root CA certificate passes COER decode and conforms to profile 9.1 | Automated ASN.1 decode + profile check |
| AC-02 | EA certificate is verifiable against Root CA public key | ECDSA signature verification test |
| AC-03 | AA certificate is verifiable against Root CA public key | ECDSA signature verification test |
| AC-04 | EC issued by EA is verifiable against EA public key | ECDSA signature verification test |
| AC-05 | AT issued by AA is verifiable against AA public key; id = none | ECDSA + field constraint test |
| AC-06 | CAM signed with AT passes SignedData verification per CAM profile | End-to-end signing and verification |
| AC-07 | DENM signed with AT includes generationLocation and signer=certificate | Field presence test |
| AC-08 | AES-CCM encrypted message decrypts correctly with ECIES key recovery | Encrypt/decrypt round-trip test |
| AC-09 | Implicit certificate reconstruction produces correct verification key | Reconstruction + ECDSA verify test |
| AC-10 | Region constraint 65535 is accepted and interpreted as EU-27 | Region parsing test |
| AC-11 | cracaId = 000000H and crlSeries = 0D in all generated certificates | Field value inspection |
| AC-12 | All structures decode without error using a conformant IEEE 1609.2 COER decoder | Cross-implementation interop test |
| AC-13 | PICS proforma (ETSI TS 103 096-1) conformance claims are substantiated | PICS review against test results |

# Appendix A: Certificate Issuance Workflow Summary

## A.1 PKI Initialization Sequence

Generate Root CA key pair (ECDSA P-256 or P-384)

Generate Root CA self-signed certificate (issuer = self; certIssuePermissions present; appPermissions with CRL and CTL ITS-AIDs)

Generate TLM key pair and TLM self-signed certificate (issuer = self; appPermissions with CTL ITS-AID)

Generate EA key pair (signing + encryption)

EA submits certificate request to Root CA; Root CA issues EA certificate (certIssuePermissions + encryptionKey present)

Generate AA key pair (signing + encryption)

AA submits certificate request to Root CA; Root CA issues AA certificate (certIssuePermissions + encryptionKey present)

## A.2 ITS-Station Enrolment Sequence

ITS-S generates EC key pair

ITS-S encrypts enrolment request to EA (using EA encryptionKey; EtsiTs103097Data-Encrypted)

EA validates ITS-S identity and issues Enrolment Credential (EC) signed by EA

EC id = choice name; appPermissions for certificate request message signing; certIssuePermissions absent

## A.3 Authorization Ticket Provisioning Sequence

ITS-S generates fresh AT key pair(s)

ITS-S encrypts AT request to AA (using AA encryptionKey; signed with EC inside EtsiTs103097Data-Encrypted)

AA validates EC and issues Authorization Ticket (AT) signed by AA

AT id = choice none; appPermissions for message signing; certIssuePermissions absent

ITS-S uses AT private key to sign CAMs and DENMs; includes AT digest (default) or full AT certificate in signer field